Battery Charging

- There are two ways to charge lithium batteries depending on what model glider they are in.

- If the glider is a newer G3 with an external charging port:

  - Connect the charging cable to the external port at the aft end of the glider

  - Turn on the charging block

    - The amperage reading should increase initially, up to ~12

    - The glider is done charging once the readout is less than 0.5 amps

- If the glider is a G2, the forward and aft batteries will need to be removed to charge (ex SL592)

  - Take glider apart to access both the forward and aft batteries

  - The charging cable has three direct charging cables corresponding to the aft, extended battery, and forward battery (the cable is identical for the forward and extended battery).

  - Use the charging cable adaptor to plug in to the aft battery first, and then use either of the remaining charging cables to plug into the forward battery.

  - Turn on the charging block

    - The amperage reading should increase initially, up to ~12

    - The glider is done charging once the readout is less than 0.5 amps

- When either replacing G2 batteries or after recharging G3 batteries, reset the coulomb counter to 0

  - Put m_coulomb_amphr_total 0

  - Exit reset

  - Get m_coulomb_amphr_total

- You can also check the lithium battery relative charge

  - Get m_lithium_battery_relative_charge

- Other relevant battery commands

  - m_battery

  - m_battery_inst

  - m_coulomb_amphr_total

  - m_coulomb_current

  - m_lithium_battery_relative_charge