# How to Update RIOT Software on a Slocum Glider

Last Updated: June 23, 2026

## Instructions:

- Download a pre-compiled MCUMGR from GitHub.

- Open a terminal window and cd to your downloaded MCUMGR files.

Ex: C:\Program Files (x86)\mcumgr-client-windows-x86

- Download zephyr-slocum.signed.bin.beta from this repo. Copy it to the same local directory as mcumgr-client and rename it to zephyr-slocum.signed.bin.

- Connect the MCIL6F RIOT debug cable to the DEBUG bulkhead connector on the science bay stiffener ring and connect the USB-serial connector to your computer.

- Open Tera Term (or equivalent terminal emulator) to baud rate 115200, 8-N-1. Hit <CR> and you should see the following prompt:

zaxy:~>

- Power off the RIOT subsystem and issue the reboot command:

zaxy:~> riot schedule end

zaxy:~> kernel reboot cold

- QUICKLY close Tera Term and navigate to the opened terminal window.

- Probe the board. This will halt the boot process unless a mcumgr reboot is issued or the system is power cycled. You have 2.5 seconds to issue the following command. If the uController does not receive comms during a 2.5s window, boot will continue to the application, and you’ll need to start again.

./mcumgr-client -d <serial-port> list

- With the boot interrupted, upload the software to the board. You should see a progress bar if the upload is working properly.

./mcumgr-client -d <serial-port> upload zephyr-slocum.signed.bin

- Reboot the board. This reboot will pass through the bootloader.

./mcumgr-client -d <serial-port> reset

- After issuing this command, you can open Tera Term to <serial-port> and should see zaxy boot as normal after 2.5 seconds. A hard power cycle could also be used here by turning the glider off, removing the green plug, and reconnecting the green plug again to reboot the glider.